COMPLETE  PALETTE- DATA HUBS

With the Complete Palette, we can use sensor values to control sounds and movement

Types of blocks:

- Common Palette Blocks

- Outputs (motors, lamps, sounds, etc)

- Inputs ( sensors)

- Flow ( loops, switches, etc. )

- Data ( arithmetic and variables)

- Advanced

Opening Data Hubs

Data Hub Challenge Part 1:

- Program a car whose speed is proportional to how loud you are yelling at it

- ( connect Sound Sensor output to Power input on Move Block)

- Data Hub Challenge Part II: Program to control steering using a steering wheel( motor rotation sensor)

-100 =       left

0=       straight

+100=       right

How do we make it more or less sensitive?

Hint: Use Arithmetic Blocks to multiply or     divide number

- Data Hub Challenge Part III: Program to control forward/backward speed( power) using throttle( motor rotation sensor)

-100 =       full power backwards

0=       stopped

+100=       full power forwards

Note: You need to use set and use the Direction Control +/-