BAND SAW SAFETY
· Safety glasses must be worn when using the bandsaw

· Do not use the bandsaw while wearing long sleeves, dangling jewelry, etc. Long hair should be tied back.

· Lower the blade guard to no more than 1” above the piece you are cutting.

· While cutting, be sure that your fingers are never behind the blade or inside the throat plate ( circle around the blade).  Your fingers should always be at least 2” from the blade- use a push stick to guide pieces when needed!

· Never put your fingers near the blade to remove pieces of wood when the saw is running.

· Never leave the saw running and walk away.
· Keep at least 2 feet from a bandsaw operator.
· When cutting,

· Avoid backing out of cuts

· Make short relief cuts to break up long cuts or sharply curved cuts.

· Do not make sharp turns
I have been instructed in the above safety rules and will follow them:

__________________            __________________

__________________            __________________

__________________            __________________

__________________            __________________

__________________            ___________________

__________________            ___________________

__________________            ___________________

__________________            ___________________

__________________            ___________________
